Satellite Clock Correction, TGD and the GPS Week Crossover

GNSS C++ Positioning Engine · Month 2

Hadi Heydarizadeh Shali · 15 October 2026

# 1. Why this note

The Month 2 apps evaluated each satellite at the reception time and used a simplified satellite clock. The correction pass changed three things, and this note explains each one with its formula: (1) the satellite clock correction, now evaluated at the transmit time; (2) TGD, the group-delay term a single-frequency L1 user must apply; (3) the week-crossover rule that keeps time differences correct across Sunday 00:00. All formulas follow IS-GPS-200N [1].

Everything starts from the code pseudorange model. The receiver clock adds range, the satellite clock subtracts it:

P = ρ + c·(dtrx − dtsat) + I + T + ε	(1)

ρ is the geometric range from the satellite at transmit time to the receiver at reception time, dtrx and dtsat are the receiver and satellite clock offsets [s], I and T are the ionospheric and tropospheric delays, and ε is noise plus multipath. The app moves the known satellite clock to the left side, so the solver only has to estimate position and dtrx:

P + c·dtsat = ρ + c·dtrx   (+ I + T + ε)	(2)

The left side is exactly what the code stores in o.pseudorange. Any error in dtsat goes straight into the range: 1 ns of clock error = 0.30 m.

# 2. The satellite clock correction

## 2.1 The broadcast clock polynomial

Each satellite broadcasts a second-order polynomial for its clock, plus a relativistic term the user computes (IS-GPS-200N §20.3.3.3.3.1). IS-GPS-200 calls this offset ΔtSV; the code calls it dt_sat:

dtsat(t) = af0 + af1·(t − toc) + af2·(t − toc)2 + Δtr	(3)

| Symbol | Meaning | Where it comes from |
| --- | --- | --- |
| af0 (clk_bias) | Clock offset at toc [s]. Can reach ~1 ms, i.e. ~300 km of range. | Nav record, line 0 |
| af1 (clk_drift) | Clock drift [s/s], typically 10−12–10−11 | Nav record, line 0 |
| af2 (clk_drift_rate) | Drift rate [s/s²], usually 0 for GPS | Nav record, line 0 |
| toc (time of clock) | Reference epoch of the clock polynomial | Nav record, line 0, as a calendar date → converted by tocSecondsOfWeek() |
| toe (time of ephemeris) | Reference epoch of the orbit parameters | Nav record, line 3, already in seconds of week |

The original code used af0 + af1·(tobs − toe): wrong time (reception instead of transmit), wrong reference epoch (toe instead of toc), and no af2, relativistic term or week wrap. For GPS toc and toe are normally the same instant, but toc is the defined reference.

## 2.2 The relativistic term

Δtr = F·e·√A·sin Ek,     F = −2√μ / c2 = −4.442807633 × 10−10 s/√m	(4)

The satellite clock runs faster in orbit than on the ground (weaker gravity) and slower because of its speed. The constant part, about +38 µs/day, is removed before launch by lowering the clock frequency from 10.23 MHz to 10.22999999543 MHz. The orbit is slightly elliptical, so a periodic part remains: near perigee the satellite is lower and faster, near apogee higher and slower. That part depends on the eccentricity e, √A and the eccentric anomaly Ek, which comes from Kepler's equation (solved by Newton iteration in eccentricAnomaly()):

Ek − e·sin Ek = Mk = M0 + n·tk	(5)

For e = 0.01 the amplitude is 22.9 ns ≈ 6.9 m. Leaving it out caused a 9.3 m position error in the Week 6 test. Precise clock products do not remove it either; it must always be applied.

## 2.3 Evaluating at the transmit time

The receiver time-tags the epoch at its own clock reading tobs = trx + dtrx. Dividing (1) by c, without the atmosphere:

tobs − P/c = (trx + dtrx) − (trx − ttx + dtrx − dtsat) = ttx + dtsat	(6)

The receiver clock cancels, so the transmit time is known before the receiver position is solved:

ttx = tobs − P/c − dtsat(ttx)	(7)

IS-GPS-200 writes the same thing as t = tSV − ΔtSV, where tSV = tobs − P/c is the satellite's own clock reading at transmission. The equation is circular (dtsat needs ttx, ttx needs dtsat), so computeSatGeom() starts with dtsat = 0 and iterates twice; the second pass changes the result by less than 10−15 s. The satellite position is then computed at ttx. The signal flight is 67–86 ms, and in that time the satellite moves 200–300 m in the Earth-fixed frame. Placing it at tobs instead caused 22.9 m of position error in the Week 6 test.

## 2.4 Where the receiver clock does matter: Earth rotation

While the signal travels, the Earth (and the ECEF frame) rotates. The satellite position at ttx must be rotated into the frame of the reception time, using the true flight time:

τ = trx − ttx = (tobs − dtrx) − ttx	(8)

x′ = x·cos θ + y·sin θ,   y′ = −x·sin θ + y·cos θ,   z′ = z,   θ = ωE·τ	(9)

The original used τ = P/c, which also contains c·dtrx − c·dtsat. Each millisecond of clock error rotates the satellite ~2 m too far (≈0.4 m of position). dtrx is unknown at first, so Pass 1 uses dtrx = 0 and Pass 2 uses Pass 1's estimate (buildObservations(dt_rx)).

| Quantity | Original (Month 2) | Corrected (_crctd) |
| --- | --- | --- |
| Satellite position | computeSatPosECEF(eph, t_obs) | at ttx from (7) |
| Satellite clock | af0 + af1(tobs − toe) | (3) at ttx, from toc, + af2 + Δtr, week-wrapped |
| Sagnac flight time | P/c (contains both clocks) | (tobs − dtrx) − ttx, two passes |

# 3. TGD — the L1/L2 group delay

## 3.1 What it is

Each signal passes through slightly different hardware inside the satellite, so the L1 and L2 codes leave the antenna at slightly different times. The broadcast clock polynomial (3) is defined for the L1/L2 ionosphere-free combination of the P(Y) codes. A user of a single frequency must shift it to that frequency with TGD, broadcast in the LNAV message (RINEX nav line 6, eph.TGD). It is a code (group) delay: it applies to pseudoranges, not to carrier phase.

## 3.2 Formulas (IS-GPS-200N §20.3.3.3.3.2)

dtsat,L1 = dtsat − TGD	(10)

dtsat,L2 = dtsat − γ·TGD,     γ = (fL1 / fL2)2 = (1575.42 / 1227.60)2 = (77/60)2 = 1.6469	(11)

TGD is typically a few nanoseconds, i.e. up to ~1–3 m of range per satellite. In the corrected app (single-frequency C1C) the solver input becomes P + c·(dtsat − TGD).

## 3.3 Why the ionosphere-free combination does not need it

The Month 3 dual-frequency solution used PIF = K1·P1 − K2·P2 with K1 = γ/(γ−1) = 2.546 and K2 = 1/(γ−1) = 1.546. Insert (10) and (11) into the satellite-clock part:

K1(dtsat − TGD) − K2(dtsat − γTGD) = (K1 − K2)·dtsat − (K1 − γK2)·TGD = dtsat	(12)

because K1 − K2 = 1 and K1 = γ·K2. TGD cancels exactly; that is why the broadcast clock is referenced to this combination.

| Observable | Apply TGD? | Why |
| --- | --- | --- |
| Single-frequency L1 code (C1C) — Weeks 6–7 | Yes, (10) | Broadcast clock is not referenced to L1 alone |
| Single-frequency L2 code | Yes, (11) with γ | Same, scaled by γ |
| Ionosphere-free code (C1W/C2W) — Month 3 | No | Cancels exactly, (12) |
| Carrier phase, double differences — Month 4 | No | Phase biases go into the ambiguities and cancel in DD |

Caveat. TGD is defined for the P(Y) code. L1 C/A (C1C) differs from L1 P(Y) by a further small inter-signal bias (ISCL1C/A, broadcast only in the modernized CNAV message [2]), usually well under a meter. Broadcast-only SPP applies TGD and ignores this remainder.

# 4. The GPS week crossover ("rollover")

## 4.1 The problem

GPS time is kept as a week number plus seconds of week, 0 … 604 800 s. The seconds counter resets to 0 every Sunday 00:00 GPS time. Every time difference in the satellite equations — t − toc in (3) and tk = t − toe in the orbit — is a difference of two seconds-of-week values. If the two times sit on opposite sides of the reset, the naive difference is wrong by one full week.

Analogy — a 12-hour clock. It is 1 o'clock, something happened at 11 o'clock. Naive: 1 − 11 = −10 h. Real: 2 h ago. Fix: add one full cycle, −10 + 12 = +2 h. For GPS the cycle is one week.

## 4.2 The rule (IS-GPS-200N, Table 20-IV)

Δt = t − tref

if  Δt > 302 400 s:     Δt ← Δt − 604 800 s	(13)

if  Δt < −302 400 s:    Δt ← Δt + 604 800 s

Why half a week: an ephemeris is only used within a few hours of toe/toc (4-hour fit interval). A real difference is never close to 3.5 days, so anything larger can only mean the counter wrapped.

| Case | t [s] | toc [s] | Naive t − toc | Wrapped | Meaning |
| --- | --- | --- | --- | --- | --- |
| Just after Sunday 00:00 | 0 | 597 600 (Sat 22:00) | −597 600 s | +7 200 s | toc was 2 h ago |
| Just before Sunday 00:00 | 604 000 (Sat 23:46) | 0 (next Sun 00:00) | +604 000 s | −800 s | toc is 13 min ahead |

Why it matters: Δt multiplies af1. Without the wrap, a typical af1 ≈ 10−11 s/s × 597 600 s ≈ 6 µs ≈ 1.8 km of range error, and the orbit would be propagated a whole week off. In the code the rule is wrapWeek() (used for t − toc and inside eccentricAnomaly()); computeSatPosECEF() in satellites.cpp (lines 151–152) already applied it to tk. For BILL at Sunday 12:00 (t = 43 200 s) it never triggers; it only matters near Saturday/Sunday midnight.

## 4.3 Not the same as the week-number rollover

The legacy LNAV message broadcasts the week number in 10 bits (0–1023), so it wraps every 1024 weeks ≈ 19.6 years: 22 Aug 1999, 7 Apr 2019, and next on 21 Nov 2038 [3]. The modernized CNAV message uses 13 bits. RINEX navigation files store the full, continuous week number, so this engine is not affected; a receiver decoding raw LNAV bits must resolve it.

# 5. Implementation map and results

Putting (2), (3), (7), (9) and (10) together, the measurement the solver fits for each satellite is:

P + c·(dtsat(ttx) − TGD) = ‖ R3(ωEτ)·rsat(ttx) − rrx ‖ + c·dtrx	(14)

| Step | Equation | Code (_crctd apps) |
| --- | --- | --- |
| Transmit time | (7) | computeSatGeom(), steps 1 and 3 |
| Clock polynomial at toc | (3) | computeSatGeom() step 2, tocSecondsOfWeek() |
| Relativistic term | (4), (5) | computeSatGeom() step 2, eccentricAnomaly() |
| Week crossover | (13) | wrapWeek(); satellites.cpp lines 151–152 |
| TGD | (10) | s.tgd = eph->TGD, o.pseudorange = P + c·(dt_sat − tgd) |
| Earth rotation | (8), (9) | buildObservations(dt_rx), Pass 1 then Pass 2 |

| Test | Original | Corrected |
| --- | --- | --- |
| Week 6 Day 4 — synthetic BILL, exact light-time truth (3D error) | 24.5 m | ≤ 1 mm |
| of which: satellite at wrong time / no relativistic term / P/c Sagnac | 22.9 m / 9.3 m / 0.4 m per ms | — |
| Week 6 Day 4 — real BILL, horizontal error | 34.4 m | 6.9 m |
| Week 7 Day 3 — real BILL, Pass 2 σ0 (8 sats, PDOP 2.156, incl. TGD) | 34.54 m | 0.61 m |

σ0 measures how well the satellites agree with each other, not accuracy: the common part of the ionosphere and troposphere is absorbed into dtrx and the height. Klobuchar and Saastamoinen (later in Week 7) target that part.

## References

[1] IS-GPS-200N, NAVSTAR GPS Space Segment / Navigation User Segment Interfaces, 1 Aug 2022: §20.3.3.3.3.1 (SV clock correction), §20.3.3.3.3.2 (L1 or L2 correction), Table 20-IV (user equations). https://www.gps.gov/technical/icwg/IS-GPS-200N.pdf

[2] Inside GNSS, "Making Sense of GPS Inter-Signal Corrections". https://insidegnss.com/making-sense-of-gps-inter-signal-corrections/

[3] EURAMET, "GPS week rollover". https://www.euramet.org/publications-media-centre/news/news/gps-week-rollover